1.

SHR: Shift right and fill vacant with 0

SAR: Shift arithmetic, fill vacant with the sign

ROL: Rotate Left, bits that falls off to the left is added to the right.

mov al, 0D4h ; EAX = 0019FFD4 ; 1101 0100

shr al, 1    ; EAX = 0019FF6A ; 0110 1010

mov al, 0D4h ; EAX = 0019FFD4 ; 1101 0100

sar al, 1    ; EAX = 0019FFEA ; 1110 1010

mov al, 0D4h ; EAX = 0019FFD4 ; 1101 0100

sar al, 4    ; EAX = 0019FFFD ; 1111 1101

mov al, 0D4h ; EAX = 0019FFD4 ; 1101 0100

rol al, 1    ; EAX = 0019FFA9 ; 1010 1001

2.

STC: Set carry flag to 1

ROR: Rotate Right, bits that falls off to the right is added to the right.

mov al, 0D4h ; EAX = 0019FFD4

ror al, 3    ; EAX = 0019FF9A

mov al, 0D4h ; EAX = 0019FFD4

rol al, 7    ; EAX = 0019FF6A

stc

mov al, 0D4h ; EAX = 0019FFD4

rcl al, 1    ; EAX = 0019FFA9

stc

mov al,0D4h  ; EAX = 0019FFD4

rcr al, 3    ; EAX = 0019FF3A

3.

222h * 100h = 0x22200h

The lower 16 bits(0x2200) will be in ax.

The upper 16 bits(0x0002) will be in dx.

mov dx, 0

mov ax, 222h

mov cx, 100h

mul cx

4.

63h / 10h = 6h

Ax is 06h

mov ax, 63h

mov bl, 10h

div bl

5.

123400h / 10h = 12340h

EAX is 12340h

mov eax, 123400h

mov edx, 0h

mov ebx, 10h

div ebx

6.

Dividend is 05004000h

05004000h / 10h = 00500400h

This is bigger than ax can hold, therefore integer overflow.

Either use bigger divisor or use bigger registry

mov ax, 4000h

mov dx, 500h

mov bx, 10h

div bx

1. write a sequence of shift instructions that cause AX to be sign extended into EAX. In other words, the sign bit of ax is copied into the upper 16 bits of EAX. Do not use the CWD instruction

mov ax, 60h

shl eax, 16; shifting EAX to the left, effectively wiping the upper 16 bits

sar eax, 16; Arithmetic shift right by 16, filling the upper 16 bits of EAX with the sign bit.

3.

Every shift basically multiplies by 2. The number of shift multiplies by 2^n. sqrt(16) = 4

mov eax, 60h

shl eax, 4

4.

Likewise, division means shifting right by 2^n = 16.

mov eax, 600h

shr eax, 4

5.

So if dl = 50h, it will be 05h afterward.

ror dl, 4    ; Rotate DL right by 4 bits

6.

shld dx, ax, 1: This shifts the bits in DX to the left by 1 bit and simultaneously shifts the most significant bit (MSB) of AX into the least significant bit (LSB) of DX.

shld dx, ax, 1

7.